# Layer 3: Scope & Definitions

# Layer 3: Shipping Activity

### Layer Objective

Identify and visualize major shipping corridors, routes, and high-traffic areas to uncover where marine vessel activity is most concentrated.

### Layer Importance

Shipping routes and traffic patterns provide a spatial proxy for marine fuel consumption, enabling the identification of high-priority regions for methanol bunkering capacity and methane-to-methanol deployment. Vessel-type-specific density data helps identify ship classes with the greatest likelihood of low-carbon methanol adoption.

### Key Questions

- Where is the highest ship traffic density (transit frequency)?

- How does traffic vary by vessel type (commercial, passenger, all vessels)?

- How close are high-traffic routes to ports capable of or planning methanol bunkering?

- Where do routes intersect with major port clusters, indicating high fuel demand?

### Stakeholder Interest

- Marine shipping companies: opportunity to understand general maritime traffic pattern and whether certain sites, based on activity, are likely to adopt alternative fuel

- Ports: opportunity to invest in low-carbon methanol infrastructure (e.g. methanol bunkering service) given justified shipping traffic

- Marine fuel producers: opportunity to identify and invest in new low-carbon methanol production sites based on traffic and viable vessels

- Marine decarbonization organizations: opportunity to prioritize marketing to specific maritime industries and vessel operators in high-traffic areas

- Methanol industry organizations: opportunity to assess market size and growth of methanol maritime fuel given shipping activity trends and methanol-capable vessels

### Definitions

- Shipping Route: A commonly used maritime path connecting major ports, reflecting an established trade pattern. In this dataset, routes are dissolved by importance tier (Major / Middle / Minor).

- Green Shipping Corridor (GSC): See Layer 2 (Relevant Regulations) for the authoritative definition. GSCs are treated as a regulatory / policy artefact rather than a general traffic pattern.

- Vessel Density: The number of vessel positions recorded per pixel over the measurement period, from AIS-derived counts. Higher values indicate more concentrated activity.

- Vessel Types: The classification of a ship based on its cargo or function. This dataset distinguishes Commercial (bulk carrier, container, tanker, general cargo, tug/offshore, etc.), Passenger (passenger ship, ro-ro/passenger ship), and Global (all classes combined).

- Commercial: Bulk carrier, general cargo, tug, offshore supply, container, oil/chemical/products/crude/LPG tanker, ro-ro/container, ore carrier, and other commercial cargo classes.

- Passenger: Passenger ship, ro-ro/passenger ship (ferries and cruise-style vessels).

- Global: All vessel types combined — the overall traffic density across every AIS-tracked class.

- Emission Control Area (ECA): See Layer 2. A designated sea area with stricter controls on airborne emissions from ships.

- Automatic Identification System (AIS): A shipboard transponder required by the IMO that provides real-time position, identification, and other information on the corresponding vessel.

- Traffic Density: The concentration of vessel activity along a shipping route (e.g. number of vessels passing per unit of time).

- Deadweight Tonnage (DWT): The maximum weight a vessel can carry.

- Twenty-foot Equivalent Unit (TEU): A standardized unit for measuring the capacity of container (cargo) vessels, based on the volume of a standard 20-foot shipping container.

### Datasets

- Shipping Routes

- Commercial Vessel Density (raster)

- Global Vessel Density (raster)

- Passenger Vessel Density (raster)

### Inclusion/Exclusion Criteria

Shipping Routes:

- Inclusion: all three tiers (Major, Middle, Minor) from the Benden 2022 Global Shipping Lanes compilation.

- Exclusion: none — the compilation is delivered whole.

Vessel Density Rasters (Commercial, Global, Passenger):

- Inclusion: every pixel with at least one recorded AIS vessel position (value ≥ 1) after zero-reclassification.

- Exclusion: pixels with a raw count of 0 (reclassified to NoData, so the density can be seen on top of a basemap in ArcGIS Online).

### Data Geometry Type

- Line — individual shipping routes (Major / Middle / Minor).

- Raster — vessel-density heatmap (one raster per vessel-type group: Commercial, Global, Passenger).

### Data Parameters

Shipping Routes:

- Object ID — "unique numeric identifier"

- Type — "Major" or "Middle" or "Minor"

- Degree — "3" (Major) or "2" (Middle) or "1" (Minor)

- Location — MultiLineString geometry (WGS 84)

Vessel Density Rasters (Commercial, Global, Passenger — identical schema):

- Pixel Value — "count of AIS vessel positions per pixel (2015–2021)"

- NoData — "0" (reclassified from zero-value pixels; rendered transparent)

- CRS — WGS 84 (EPSG:4326)

- Native Resolution — 0.005° (~550 m at the equator); 72,006 × 33,998 pixels

- Delivered Resolution — aggregated by factor of 40 (fun = "max") for GitHub size limits; native-resolution recipe in 02_Scripts/04_Clip_Vessel_Density.R